# 环境执行与控制源码解析

源码快照 2026 年 10 月 12 日

从一个策略控制步读到 PhysX 运动结果。所有 LAB 回调都在 wheel_legged_vmc_flat_env.py 中；五连杆数学和硬件标定是独立功能计算，不再通过 Mixin 继承寻找动作实现。

这里解释实际源码怎样工作，而不是仅列改动历史。片段保留原源码和行号；长行只在文档展示中换行，片段不是可直接独立运行的完整文件。当前车辆、PD/VMC、硬件上限和训练分布没有因编写文档而修改。

# 1 一次 step 的主线

策略 50 Hz 产生六维动作；_pre_physics_step 保存 raw 和执行边界；_apply_action 在每个 200 Hz 物理子步重新读状态并设置控制目标；LAB 写入缓存、推进物理，再判断终止和奖励、reset 失败环境、生成下一帧观测。PPO 不计算闭链动力学，PhysX 不学习策略。

源码 direct_rl_env.py 第 340 至 397 行

action = self._action_noise_model(action)

# process actions

self._pre_physics_step(action)

# check if we need to do rendering within the physics loop

# note: checked here once to avoid multiple checks within the loop

is_rendering = self.sim.has_gui() or self.sim.has_rtx_sensors()

# perform physics stepping

for _ in range(self.cfg.decimation):

self._sim_step_counter += 1

# set actions into buffers

self._apply_action()

# set actions into simulator

self.scene.write_data_to_sim()

# simulate

self.sim.step(render=False)

# render between steps only if the GUI or an RTX sensor needs it

# note: we assume the render interval to be the shortest accepted

rendering interval.

#    If a camera needs rendering at a faster frequency, this will

lead to unexpected behavior.

if self._sim_step_counter % self.cfg.sim.render_interval == 0 and

is_rendering:

self.sim.render()

# update buffers at sim dt

self.scene.update(dt=self.physics_dt)

# post-step:

# -- update env counters (used for curriculum generation)

self.episode_length_buf += 1  # step in current episode (per env)

self.common_step_counter += 1  # total step (common for all envs)

self.reset_terminated[:], self.reset_time_outs[:] = self._get_dones()

self.reset_buf = self.reset_terminated | self.reset_time_outs

self.reward_buf = self._get_rewards()

# -- reset envs that terminated/timed-out and log the episode

information

reset_env_ids = self.reset_buf.nonzero(as_tuple=False).squeeze(-1)

if len(reset_env_ids) > 0:

self._reset_idx(reset_env_ids)

# if sensors are added to the scene, make sure we render to reflect

changes in reset

if self.sim.has_rtx_sensors() and self.cfg.rerender_on_reset:

self.sim.render()

# post-step: step interval event

if self.cfg.events:

if "interval" in self.event_manager.available_modes:

self.event_manager.apply(mode="interval", dt=self.step_dt)

# update observations

self.obs_buf = self._get_observations()

# add observation noise

# note: we apply no noise to the state space (since it is used for

critic networks)

if self.cfg.observation_noise_model:

self.obs_buf["policy"] =

self._observation_noise_model(self.obs_buf["policy"])

# return observations, rewards, resets and extras

return self.obs_buf, self.reward_buf, self.reset_terminated,

self.reset_time_outs, self.extras

# 2 环境初始化和关节列

源码 wheel_legged_vmc_flat_env.py 第 69 至 111 行

def __init__(

self, cfg: WheelLeggedVMCFlatEnvCfg, render_mode: str | None = None,

**kwargs

):

# configclass 的 to_dict() 将嵌套配置转为字典，供车辆契约检查；不创建机器人。

cfg.five_bar_geometry = validate_robot_contract(cfg.to_dict())

# LAB 基类先创建 self.sim / self.scene，再回调本类的 _setup_scene()。

super().__init__(cfg, render_mode, **kwargs)

# single_action_space 描述「单个机器人」的动作空间；张量实际是 [N, 6]。

self._action_dim = int(self.single_action_space.shape[0])

self._validate_configuration()

# LAB find_joints() 返回「仿真关节下标、匹配名称」，而不是电机对象。

leg_joint_names = list(self.cfg.leg_joint_names)

wheel_joint_names = list(self.cfg.wheel_joint_names)

torque_joint_names = [

leg_joint_names[0], leg_joint_names[1], wheel_joint_names[0],

leg_joint_names[2], leg_joint_names[3], wheel_joint_names[1],

]

self._leg_joint_ids, resolved_legs = self._robot.find_joints(

leg_joint_names, preserve_order=True

)

self._wheel_joint_ids, resolved_wheels = self._robot.find_joints(

wheel_joint_names, preserve_order=True

)

self._torque_joint_ids, resolved_torques = self._robot.find_joints(

torque_joint_names, preserve_order=True

)

if (

resolved_legs != leg_joint_names

or resolved_wheels != wheel_joint_names

or resolved_torques != torque_joint_names

or self._action_dim != len(torque_joint_names)

):

raise ValueError(

"VMC joint/action layout does not match the robot articulation:

"

f"legs={resolved_legs}, wheels={resolved_wheels}, "

f"torques={resolved_torques}, actions={self._action_dim}"

)

print(

"[INFO] VMC joint mapping: "

f"articulation={self._robot.joint_names}, "

f"legs={self._leg_joint_ids}, wheels={self._wheel_joint_ids}, "

super().__init__ 创建 sim 与 scene，并回调 _setup_scene。随后 find_joints(..., preserve_order=True) 得到仿真列 ID。四个主动电机顺序是 LB LL RB RL，动作/力矩顺序是 [LB, LL, 左轮, RB, RL, 右轮]。不能把 CAD 的关节数组顺序直接当成策略顺序，也不能把第二个主动髋关节误认为普通串联膝关节。

| 缓存 | 形状 | 作用 |
| --- | --- | --- |
| _​raw_​actions /​ _​actions | N × 6 | 未执行限制的输出／有效动作 |
| _​last_​actions | N × 6 × 2 | 执行动作历史，不是额外五帧 obs |
| _​commands | N × 4 | vx、yaw rate、高度、可选航向 |
| _​L0 /​ _​theta0 | N × 2 | 左右虚拟腿实际状态 |
| _​five_​bar_​jacobian | N × 2 × 2 × 2 | 腿长/​摆角对主动关节角的导数 |
| _​joint_​position_​ref_​applied | N × 4 | 实际应用的四关节目标 |
| _​torque_​limits | N × 6 | 输出轴力矩限幅 |

# 3 创建执行器和完整闭链场景

源码 wheel_legged_vmc_flat_env.py 第 215 至 268 行

def _setup_scene(self):

# LAB 在基类初始化期间调用此钩子；这里构建场景，不执行策略推理。

self._external_forces_every_iteration = apply_force_iteration_setting(

self.sim.stage, self.cfg.sim.physics_prim_path,

self.cfg.enable_external_forces_every_iteration,

)

print('[INFO] PhysX external forces every iteration: '

f'{self._external_forces_every_iteration}')

# 将顶层力矩上限同步到嵌套 actuator 配置，必须先于 Articulation 初始化。

synchronize_actuator_limits(self.cfg)

leg_stiffness, leg_damping = leg_drive_gains(

self.cfg.leg_control_mode, self.cfg.leg_joint_stiffness,

self.cfg.leg_joint_damping

)

# 显式 VMC 把关节位置 PD 设为 0/0；关节角模式则启用配置的 PD。

self.cfg.robot.actuators['legs'].stiffness = leg_stiffness

self.cfg.robot.actuators['legs'].damping = leg_damping

self._robot = Articulation(self.cfg.robot)

from pxr import UsdPhysics

for name in self.cfg.leg_joint_names:

# 下面是 USD 底层 API，不是读取机器人当前角度；Prim 表示场景中的节点。

prim=self.sim.stage.GetPrimAtPath(f'/World/envs/env_0/Robot/joints/{name}')

if not prim.IsA(UsdPhysics.RevoluteJoint):

raise ValueError(f'Leg drive joint not found: {name}')

drive=UsdPhysics.DriveAPI.Apply(prim,'angular')

# Apply 给该关节附加角驱动 schema；Create*Attr 写入驱动配置属性。

drive.CreateTypeAttr('force')

drive.CreateStiffnessAttr(leg_stiffness)

drive.CreateDampingAttr(leg_damping)

drive.CreateMaxForceAttr(self.cfg.leg_effort_limit)

if self.cfg.leg_control_mode == DIRECT_JOINT_POSITION_MODE:

# 普通 maxForce 仅限制驱动 PD；PhysX envelope 同时约束 PD 和前馈。

from omni.physx.bindings._physx import PERF_ENV_API

if not prim.ApplyAPI(PERF_ENV_API, 'angular'):

raise RuntimeError('Cannot enforce combined joint PD +

feedforward effort limit')

if not prim.HasAPI(PERF_ENV_API, 'angular'):

raise RuntimeError('Combined joint effort envelope was not

applied')

print(f'[INFO] Leg execution: {self.cfg.leg_control_mode}; '

f'joint-drive stiffness={leg_stiffness}, damping={leg_damping}.')

if self.cfg.wheel_control_mode == "implicit_velocity":

from pxr import UsdPhysics

# 原 CAD 没有导入器驱动；只为两轮创建所需的速度阻尼驱动。

for name in self.cfg.wheel_joint_names:

path = f"/World/envs/env_0/Robot/joints/{name}"

prim = self.sim.stage.GetPrimAtPath(path)

if not prim.IsA(UsdPhysics.RevoluteJoint):

raise ValueError(f"Wheel drive joint not found: {path}")

drive = UsdPhysics.DriveAPI.Apply(prim, "angular")

drive.CreateTypeAttr("force")

drive.CreateStiffnessAttr(0.0)

drive.CreateDampingAttr(self.cfg.wheel_damping)

drive.CreateMaxForceAttr(self.cfg.wheel_effort_limit)

drive.CreateTargetVelocityAttr(0.0)

# 注册到 LAB scene，后续 scene.write_data_to_sim()/update()/reset()

self.scene.articulations["robot"] = self._robot

Articulation 把 USD 导入场景；scene.articulations 注册后，LAB 才统一处理 write/update/reset。隐式关节模式启用位置 PD，并给主动关节应用 combined effort envelope；显式 VMC 设置位置 Drive Kp/Kd 为 0/0。轮子 stiffness=0，damping 对速度误差产生力矩，不控制无界累计轮角。被动闭链电机组必须零驱动。

ContactSensor 的 body ID 与关节 ID 不同。relocate_collision_groups 保留新车已验证的碰撞组；clone_environments 从 env_0 建立各实例。TerrainImporter 的 env_origins 用于各机器人 reset 的世界坐标，DomeLight 只负责照明。

# 4 策略输出和执行动作边界

源码 wheel_legged_vmc_flat_env.py 第 289 至 294 行

def _pre_physics_step(self, actions: torch.Tensor):

# 高斯动作可能越界；保留 raw 给饱和奖励，执行动作限制到 [-1,1]。

self._raw_actions = torch.nan_to_num(

actions, nan=0.0, posinf=1.0, neginf=-1.0

)

self._actions = torch.clamp(self._raw_actions, -1.0, 1.0)

actor 输出高斯均值，训练时采样可能越过 [-1,1]。保留 raw 的原因是软饱和奖励和诊断；执行前仍必须裁剪。PPO clip_param 限制新旧动作概率比，不负责限制电机命令。NaN/Inf 的输入保护不能代替数值故障终止。

# 5 直接关节目标角控制

源码 wheel_legged_vmc_flat_env.py 第 385 至 417 行

def _apply_direct_joint_position_action(self):

"""Four active motor positions through PhysX joint PD, plus unchanged

wheels."""

target, applied = joint_position_targets(

self._raw_actions[:, LEG_ACTION_COLUMNS],

self.cfg.leg_joint_position_offset,

self.cfg.leg_joint_position_scale,

self._robot.data.joint_pos_limits[:, self._leg_joint_ids],

self.cfg.leg_joint_target_limit_margin, validate=False,

)

self._actions[:, LEG_ACTION_COLUMNS] = applied

self._joint_position_ref_applied = target

self._robot.set_joint_position_target(target,

joint_ids=self._leg_joint_ids)

self._robot.set_joint_velocity_target(torch.zeros_like(target),

joint_ids=self._leg_joint_ids)

# 直接关节模式的显式 effort 只含重力前馈，不能叠加另一套 VMC 反馈。

torques = torch.zeros_like(self._actions)

if self.cfg.leg_joint_gravity_feedforward:

t1, t2 = five_bar_torques(

self._five_bar_jacobian, self._get_gravity_feedforward(),

torch.zeros_like(self._theta0),

)

gravity_effort = torch.stack((t1[:, 0], t2[:, 0], t1[:, 1], t2[:,

1]), dim=-1)

self._leg_gravity_effort_applied =

gravity_effort.clamp(-self.cfg.leg_effort_limit, self.cfg.leg_effort_limit)

else:

self._leg_gravity_effort_applied.zero_()

torques[:, LEG_ACTION_COLUMNS] = self._leg_gravity_effort_applied

wheel_ref = self._actions[:, (2, 5)] * self.cfg.action_scale_vel

wheel_effort = self._wheel_effort_from_reference(wheel_ref)

if self.cfg.wheel_control_mode != 'implicit_velocity':

torques[:, (2, 5)] = wheel_effort * wheel_effort.new_tensor([1.,

-1.])

self._robot.set_joint_effort_target(

torques.clamp(-self._torque_limits, self._torque_limits),

joint_ids=self._torque_joint_ids,

)

# FK of motor targets is telemetry only, never a control reference map.

length, angle, _, _ = five_bar_state(target[:, ::2], target[:, 1::2],

**self.cfg.five_bar_geometry)

self._l0_ref_applied = length

self._theta0_ref_applied = angle

源码 leg_control.py 第 21 至 51 行

def joint_position_targets(actions, offset, scale, limits, margin=0.0, *,

validate=True):

"""Four normalized policy coordinates -> raw CAD motor angles [rad].

No virtual-leg reference, IK, height feedback or explicit joint PD

participates in this position map. Gravity feedforward is a separate

effort channel, never an angle offset. Limits are the articulation's

hard position limits, intersected with the configured action envelope.

"""

# 纯张量函数，不调用 LAB/PhysX：调用方传入 Articulation.data.joint_pos_limits。

# actions 为 [N, 4]，limits 为 [N, 4, 2]；四列顺序是 LB、LL、RB、RL。

import torch

if actions.ndim != 2 or actions.shape[1] != 4:

raise ValueError('Expected a batch of four active-joint actions')

if validate:

validate_joint_position_map(offset, scale, margin)

if limits.shape != (*actions.shape, 2):

raise ValueError('Joint limits must have shape [num_envs, 4, 2]')

if validate and (not torch.isfinite(limits).all() or not ((limits[..., 1] -

limits[..., 0]) > 2 * margin).all()):

raise ValueError('Joint limits must be finite and wider than twice the

safety margin')

center, amplitude = actions.new_tensor(offset), actions.new_tensor(scale)

# 允许区间是「nominal±动作尺度」与「USD 关节角限位减 margin」的交集。

# 全部用 rad；它不是 CAD 限位块接触包络，也不是虚拟腿长的硬限制。

lower = torch.maximum(limits[..., 0] + margin, center - amplitude)

upper = torch.minimum(limits[..., 1] - margin, center + amplitude)

if validate and (not (lower < upper).all() or not ((center >= lower) &

(center <= upper)).all()):

raise ValueError('Joint action envelope and safe articulation limits

must contain the reference pose')

normalized = torch.nan_to_num(actions, nan=0., posinf=1.,

neginf=-1.).clamp(-1., 1.)

target = torch.maximum(torch.minimum(center + normalized * amplitude,

upper), lower)

# 第二项将最终角度反算为有效归一化动作，供 last_actions 和奖励使用。

# 这里只生成参考；LAB set_joint_position_target() 和隐式 PD 在环境中执行。

return target, (target - center) / amplitude

目标角 = nominal offset + clip(raw action) × scale。再与 USD 物理关节边界减 margin 的范围取交集，并反算实际应用的归一化 action。0 action 是 nominal pose，不是失能，也不是零力矩。高度命令只作为策略输入，不经过 IK 或外层高度控制器自动改写目标角。

set_joint_position_target 写角度参考，set_joint_velocity_target 给腿部零速度参考。反馈力矩由 PhysX 隐式 PD 求解；环境只在显式 effort 通道附加 Jᵀ[F_gravity,0]。执行器总输出受同一关节力矩包络约束。函数末尾对目标角做 FK 是记录诊断参考，不是另一条控制链。

# 6 轮速度和镜像方向

源码 wheel_legged_vmc_flat_env.py 第 419 至 426 行

def _wheel_effort_from_reference(self, reference):

"""Shared forward-positive wheel controller for both leg modes."""

if self.cfg.wheel_control_mode == 'implicit_velocity':

self._robot.set_joint_velocity_target(

reference * reference.new_tensor([1., -1.]),

joint_ids=self._wheel_joint_ids,

)

return torch.zeros_like(reference)

return self.cfg.wheel_damping * (reference -

self._get_wheel_vel_forward_positive())

源码 wheel_legged_vmc_flat_env.py 第 555 至 565 行

def _get_wheel_vel_forward_positive(self) -> torch.Tensor:

"""Return wheel speeds with positive values meaning forward motion.

The mirrored URDF hip frames make the raw right-wheel joint axis point

opposite to the raw left-wheel joint axis. Canonicalizing the right

coordinate here gives the policy one consistent wheel convention.

"""

wheel_vel_raw = self._robot.data.joint_vel[:, self._wheel_joint_ids]

return torch.stack(

(wheel_vel_raw[:, 0], -wheel_vel_raw[:, 1]), dim=1

)

两轮归一化动作乘 15 得到 rad/s 参考。读状态时把原始右轮速度翻一次符号；写参考时再转换回原始坐标。两处是在两个边界做互逆转换，不是重复错误取反。零速平衡时控制器依然需要非零轮力矩，不能把“静止”实现成强制零力矩。

# 7 显式 VMC 路径

源码 wheel_legged_vmc_flat_env.py 第 296 至 383 行

def _apply_action(self):

"""Apply this policy's configured actuator contract.

Called once per physics substep (200 Hz). Mirrors the original

`leg_post_physics_step` + `_compute_torques` inside the decimation loop.

"""

self._update_forward_kinematics()

if self.cfg.leg_control_mode == DIRECT_JOINT_POSITION_MODE:

self._apply_direct_joint_position_action()

return  # Direct motor references; gravity was handled above.

# reference targets from actions

theta0_ref = self._actions[:, (0, 3)] * self.cfg.action_scale_theta

# Keep the virtual-leg angle target inside its physical safety range

theta0_ref = torch.clamp(

theta0_ref,

min=self.cfg.theta0_ref_min,

max=self.cfg.theta0_ref_max,

)

# 每个子步从 raw 重建长度参考，避免对已修正的 _actions 重复叠加高度修正。

l0_ref = (

torch.clamp(self._raw_actions[:, (1, 4)], -1.0, 1.0)

* self.cfg.action_scale_l0

+ self.cfg.l0_offset

)

nominal_l0_ref = self._commands[:, 2:3] +

self.cfg.leg_length_height_offset

l0_ref = (

(1.0 - self.cfg.height_reference_blend) * l0_ref

+ self.cfg.height_reference_blend * nominal_l0_ref

)

height_error = self._commands[:, 2] - self._robot.data.root_pos_w[:, 2]

height_correction = torch.clamp(

height_error * self.cfg.height_feedback_gain,

min=-self.cfg.height_feedback_max_adjustment,

max=self.cfg.height_feedback_max_adjustment,

)

l0_ref = l0_ref + height_correction[:, None]

# 高度混合和外环修正后再限制腿长，动作限幅不能替代这一层参考限幅。

l0_ref = torch.clamp(

l0_ref, min=self.cfg.l0_ref_min, max=self.cfg.l0_ref_max

)

forward_support = (

self._commands[:, 0] >= self.cfg.forward_support_speed_threshold

)

support_floor = torch.minimum(

torch.full_like(height_error,

self.cfg.forward_support_min_leg_length),

self._commands[:, 2] + self.cfg.forward_support_height_margin,

)

l0_ref = torch.where(

forward_support[:, None],

torch.maximum(l0_ref, support_floor[:, None]),

l0_ref,

)

# 固定控制器和导轨诊断可覆写这两个钩子；普通平地控制不额外改参考。

l0_ref, theta0_ref = self._modify_leg_references(l0_ref, theta0_ref)

self._l0_ref_applied = l0_ref

self._theta0_ref_applied = theta0_ref

self._actions[:, (1, 4)] = (

l0_ref - self.cfg.l0_offset

) / self.cfg.action_scale_l0

wheel_vel_ref = self._actions[:, (2, 5)] * self.cfg.action_scale_vel

# VMC leg control (PD on virtual coordinates theta0 / L0)

torque_leg = (

self.cfg.kp_theta * (theta0_ref - self._theta0)

- self.cfg.kd_theta * self._theta0_dot

)

force_leg = (

self.cfg.kp_l0 * (l0_ref - self._L0) - self.cfg.kd_l0 * self._L0_dot

)

torque_wheel = self._wheel_effort_from_reference(wheel_vel_ref)

# map virtual forces/torques to actual joint torques (closed chain)

force_bias = self._get_leg_force_bias()

T1, T2 = self._vmc(force_leg + self._get_gravity_feedforward() +

force_bias, torque_leg)

torques = torch.stack(

(

T1[:, 0], T2[:, 0], torque_wheel[:, 0],

T1[:, 1], T2[:, 1],

# 写入边界将前进为正的右轮力矩转换回镜像关节坐标。

-torque_wheel[:, 1],

),

dim=1,

)

torques = torch.clip(torques, -self._torque_limits, self._torque_limits)

self._robot.set_joint_effort_target(torques,

joint_ids=self._torque_joint_ids)

VMC 模式把 0/3 列映射为两侧虚拟摆角，1/4 列映射为虚拟腿长，2/5 列映射为轮速。摆角、腿长各有物理参考边界；高度混合、受限高度误差修正和高速支撑下界只作用于这个分支。虚拟摆角 PD 给 T，腿长 PD 加重力给 F，再经 Jᵀ[F,T] 转成每侧两个主动关节 effort。

_modify_leg_references 的默认结果不改轨迹，_get_leg_force_bias 默认为零；保留的是固定控制器数值验证钩子，不是旧 IK 或跳跃状态机。当前平地策略没有新增起跳行为。

# 8 正解和虚功映射

源码 wheel_legged_vmc_flat_env.py 第 442 至 461 行

def _update_forward_kinematics(self, env_ids: torch.Tensor | None = None):

"""Mapped large five-bar state, Jacobian and analytic virtual

velocity."""

dof_pos = self._robot.data.joint_pos

dof_vel = self._robot.data.joint_vel

if env_ids is not None:

dof_pos = dof_pos[env_ids]

dof_vel = dof_vel[env_ids]

q_b = dof_pos[:, self._leg_joint_ids[::2]]

q_l = dof_pos[:, self._leg_joint_ids[1::2]]

length, angle, jacobian, valid = five_bar_state(q_b, q_l,

**self.cfg.five_bar_geometry)

velocity = torch.stack((dof_vel[:, self._leg_joint_ids[::2]],

dof_vel[:, self._leg_joint_ids[1::2]]), dim=-1)

virtual_velocity = (jacobian @ velocity[..., None]).squeeze(-1)

selection = slice(None) if env_ids is None else env_ids

self._L0[selection] = length

self._theta0[selection] = angle

self._L0_dot[selection] = virtual_velocity[..., 0]

self._theta0_dot[selection] = virtual_velocity[..., 1]

self._five_bar_jacobian[selection] = jacobian

self._five_bar_valid[selection] = valid

源码 five_bar_vmc.py 第 44 至 47 行

def five_bar_torques(jacobian, force, torque):

"""Virtual work in raw motor coordinates, including each side's axis

sign."""

effort=jacobian.transpose(-1,-2)@torch.stack((force,torque),dim=-1)[...,None]

return effort[...,0,0],effort[...,1,0]

q_B 与 q_L 是每侧两个主动电机角。five_bar_state 计算轮端 L0、theta0、Jacobian 和有效性；虚拟速度等于 Jacobian 乘主动关节速度。J 的行对应腿长和摆角，列对应 B/L 主动电机，因此力矩映射应与虚功一致。闭链分支错误或接近奇异会触发终止，不能靠 PPO 学会掩盖错误运动学。

# 9 回合重置和 LAB 状态缓存

源码 wheel_legged_vmc_flat_env.py 第 1074 至 1149 行

def _reset_idx(self, env_ids: Sequence[int] | None):

# env_ids 选择「要重置的机器人」，不是关节下标；未给出时重置全部实例。

if env_ids is None:

env_ids = self._robot._ALL_INDICES

terminated_command_x = self._commands[env_ids, 0].clone()

terminated_target_x = self._target_lin_vel_x[env_ids].clone()

terminated_command_height = self._commands[env_ids, 2].clone()

resample_steps = max(int(self.cfg.commands.resampling_time /

self.step_dt), 1)

steps_since_resample = self.episode_length_buf[env_ids].clone() %

resample_steps

# Articulation.reset() 清理执行器等缓存，并不会把刚体瞬移回初始姿态。

self._robot.reset(env_ids)

super()._reset_idx(env_ids)

# .data.default_* 是初始模板；clone 后修改，避免污染其他 env 的默认状态。

joint_pos = self._robot.data.default_joint_pos[env_ids].clone()

joint_vel = torch.zeros_like(joint_pos)

# root_state 为 [位置 xyz, 四元数 wxyz, 世界线速度 xyz, 世界角速度 xyz] 共 13 维。

default_root_state =

self._robot.data.default_root_state[env_ids].clone()

# 默认位置是相对各 env 原点的；加上 env_origins 才是世界坐标位置。

default_root_state[:, :3] += self._terrain.env_origins[env_ids]

# 训练重置扰动随课程放大；固定命令回放使用配置的最终扰动幅度。

fixed_command = float(self._command_ranges[0, 0].item()) == float(

self._command_ranges[0, 1].item()

)

progress = 1.0 if fixed_command else min(max(

(self.common_step_counter - self.cfg.commands.standing_only_steps)

/ max(self.cfg.commands.motion_ramp_steps, 1), 0.0

), 1.0)

reset_velocity = self.cfg.reset_velocity_initial + progress * (

self.cfg.reset_velocity_final - self.cfg.reset_velocity_initial

)

default_root_state[:, 7:13] = (

2.0 * torch.rand(len(env_ids), 6, device=self.device) - 1.0

) * reset_velocity

# LAB write_*_to_sim() 直接写入物理状态和相关缓存；不是 PD 的目标参考。

self._robot.write_root_pose_to_sim(default_root_state[:, :7], env_ids)

self._robot.write_root_velocity_to_sim(default_root_state[:, 7:],

env_ids)

# 第三个参数 joint_ids=None 表示所有关节；仅作用于 env_ids 选中的实例。

self._robot.write_joint_state_to_sim(joint_pos, joint_vel, None,

env_ids)

self._update_forward_kinematics(env_ids)

# reset buffers

self._raw_actions[env_ids] = 0.0

self._actions[env_ids] = 0.0

self._last_actions[env_ids] = 0.0

self._joint_position_ref_applied[env_ids] = 0.0

self._l0_ref_applied[env_ids] = 0.0

self._theta0_ref_applied[env_ids] = 0.0

self._leg_gravity_effort_applied[env_ids] = 0.0

if self.cfg.leg_control_mode == DIRECT_JOINT_POSITION_MODE:

# 状态复位不等于目标清零：还要清理旧参考和前馈，避免新回合继承饱和动作。

self._joint_position_ref_applied[env_ids] =

self._joint_position_ref_applied.new_tensor(

self.cfg.leg_joint_position_offset,

)

# LAB set_joint_*_target() 写目标缓存，下一次 write_data_to_sim() 才应用。

self._robot.set_joint_position_target(

self._joint_position_ref_applied[env_ids],

joint_ids=self._leg_joint_ids, env_ids=env_ids,

)

self._robot.set_joint_velocity_target(

torch.zeros((len(env_ids), 4), device=self.device),

joint_ids=self._leg_joint_ids, env_ids=env_ids,

)

self._robot.set_joint_effort_target(

torch.zeros((len(env_ids), 6), device=self.device),

joint_ids=self._torque_joint_ids, env_ids=env_ids,

)

self._fail_buf[env_ids] = 0.0

self._base_contact_buf[env_ids] = 0.0

self._pitch_fail_buf[env_ids] = 0.0

self._roll_fail_buf[env_ids] = 0.0

self._leg_contact_buf[env_ids] = 0.0

self._low_height_buf[env_ids] = 0.0

self._commands[env_ids] = 0.0

if self.cfg.commands.heading_command:

forward_axis = torch.tensor(

[1.0, 0.0, 0.0], device=self.device

).expand(len(env_ids), -1)

# LAB quat_apply(q, v) 用 wxyz 四元数把机体系前向向量旋转到世界系。

Articulation.reset 清缓存，不会把机器人瞬移回初始姿态；write_root_pose/velocity 和 write_joint_state 才写入物理状态。default_* 先 clone 再修改，避免污染其他 env。重置只清选中的机器人，动作历史、连续失败计数、目标角和前馈也要同时清除，防止下一回合继承饱和参考。extras 日志归一化不参与控制。

# 源码索引和版本核对

IsaacLab/source/isaaclab/isaaclab/envs/direct_rl_env.py

wheel_legged_isaaclab/wheel_legged_gym_isaaclab/five_bar_vmc.py

wheel_legged_isaaclab/wheel_legged_gym_isaaclab/leg_control.py

wheel_legged_isaaclab/wheel_legged_gym_isaaclab/tasks/direct/wheel_legged_vmc_flat/wheel_legged_vmc_flat_env.py

版本核对不应只看文件名。模型应携带 params/env.yaml 和 params/agent.yaml；训练分支的覆盖值以配套配置为准。本教程解释源码快照，不作为随训练动态变化的实时监控面板。

库源码来自本地 Isaac Lab 和 RSL-RL 的 BSD-3-Clause 版本；对应源码保留 Isaac Lab、ETH Zurich 和 NVIDIA 等版权及许可证头。这里只摘录相关调用，不修改第三方实现。