# 观测命令奖励与终止源码解析

源码快照 2026 年 10 月 12 日

从同一个 env 类阅读 _get_observations、_get_rewards 和 _get_dones。命令抽样较长，放在 env_commands.py，但仍写当前环境的同一份缓存。重点是区分命令、实际状态、策略目标和最终力矩。

这里解释实际源码怎样工作，而不是仅列改动历史。片段保留原源码和行号；长行只在文档展示中换行，片段不是可直接独立运行的完整文件。当前车辆、PD/VMC、硬件上限和训练分布没有因编写文档而修改。

# 1 27 维观测逐段对应

| 切片 | 实际物理量 | 缩放与约定 |
| --- | --- | --- |
| 0:3 | 机体系角速度 xyz | rad/​s × 0.25 |
| 3:6 | 机体系重力方向 | 单位向量，水平 [0,0,-1] |
| 6:9 | vx /​ yaw rate /​ 高度命令 | m/​s × 2，rad/​s × 0.25，m × 5 |
| 9:13 | LB LL RB RL 实际关节角减 offset | rad × 1 |
| 13:17 | 四主动关节速度 | rad/​s × 0.05 |
| 17:19 | yaw 对齐水平机身 vx、vy | m/​s × 2，不随 pitch/​roll 倾斜 |
| 19:21 | 左右实际轮速 | rad/​s × 0.05，两轮前进为正 |
| 21:27 | 上一周期有效执行动作 | 不是 actor 未裁剪的 raw mean |

源码 wheel_legged_vmc_flat_env.py 第 469 至 536 行

def _get_observations(self) -> dict:

# command resampling (once every resampling_time seconds)

self._update_commands()

max_command_step = self.cfg.commands.linear_acceleration_limit *

self.step_dt

self._commands[:, 0] += torch.clamp(

self._target_lin_vel_x - self._commands[:, 0],

min=-max_command_step,

max=max_command_step,

)

# heading command -> yaw-rate command

if self.cfg.commands.heading_command:

forward_axis = torch.tensor(

[1.0, 0.0, 0.0], device=self.device

).expand(self.num_envs, -1)

forward = quat_apply(

self._robot.data.root_quat_w, forward_axis

)

heading = torch.atan2(forward[:, 1], forward[:, 0])

self._commands[:, 1] = torch.clip(

self.cfg.commands.heading_kp * wrap_to_pi(self._commands[:, 3] -

heading),

-self.cfg.commands.heading_rate_limit,

self.cfg.commands.heading_rate_limit,

)

# base height (flat ground)

self._base_height = self._robot.data.root_pos_w[:, 2]

# store previous actions for reward computation

self._last_actions[:, :, 1] = self._last_actions[:, :, 0]

self._last_actions[:, :, 0] = self._actions[:]

heading_velocity = self._get_heading_frame_horizontal_velocity()

wheel_velocity_forward = self._get_wheel_vel_forward_positive()

command_obs = torch.nan_to_num(

self._commands[:, :3], nan=0.0, posinf=0.0, neginf=0.0

)

obs = torch.cat(

(

self._sanitize_observation(

self._robot.data.root_ang_vel_b, self.cfg.obs_clip.ang_vel

) * self.cfg.obs_scales.ang_vel,

self._sanitize_observation(

self._robot.data.projected_gravity_b,

self.cfg.obs_clip.projected_gravity,

),

command_obs * self._commands_scale,

self._get_leg_observation(),

# 轮角连续累积而不输入 obs；保留实际机身水平速度和两轮转速。

self._sanitize_observation(

heading_velocity, self.cfg.obs_clip.lin_vel

) * self.cfg.obs_scales.lin_vel,

self._sanitize_observation(

wheel_velocity_forward,

self.cfg.obs_clip.wheel_vel,

) * self.cfg.obs_scales.dof_vel,

self._sanitize_observation(

self._actions, self.cfg.obs_clip.action

),

),

dim=-1,

)

obs = torch.nan_to_num(obs, nan=0.0, posinf=0.0, neginf=0.0)

# reset 会替换 extras['log']，在返回观测前补回本控制步的诊断统计。

if self._tracking_diagnostics:

self.extras['log'] = merge_step_diagnostics(

self.extras.get('log', {}), self._tracking_diagnostics,

)

return {"policy": obs}

torch.cat 沿最后一维拼接 [N,27]，不是沿机器人批次把数量加起来。unsqueeze 或 [:,None] 增加广播维，不凭空增加观测字段。实际高度没有单独 obs 字段，当前策略接收高度命令、关节和姿态/速度状态。VMC 模式 9:17 则是虚拟腿摆角、摆角速度、长度和长度速度，即使输入输出尺寸相同也不能加载关节角策略。

源码 wheel_legged_vmc_flat_env.py 第 538 至 553 行

def _get_leg_observation(self):

if self.cfg.leg_control_mode == DIRECT_JOINT_POSITION_MODE:

position = self._robot.data.joint_pos[:, self._leg_joint_ids]

offset = position.new_tensor(self.cfg.leg_joint_position_offset)

return torch.cat((

self._sanitize_observation(position - offset,

self.cfg.obs_clip.theta) * self.cfg.obs_scales.dof_pos,

self._sanitize_observation(

self._robot.data.joint_vel[:, self._leg_joint_ids],

self.cfg.obs_clip.theta_dot,

) * self.cfg.obs_scales.dof_vel,

), dim=-1)

return torch.cat((

self._sanitize_observation(self._theta0, self.cfg.obs_clip.theta) *

self.cfg.obs_scales.dof_pos,

self._sanitize_observation(self._theta0_dot,

self.cfg.obs_clip.theta_dot) * self.cfg.obs_scales.dof_vel,

self._sanitize_observation(self._L0, self.cfg.obs_clip.leg_length) *

self.cfg.obs_scales.l0,

self._sanitize_observation(self._L0_dot,

self.cfg.obs_clip.leg_length_dot) * self.cfg.obs_scales.l0_dot,

), dim=-1)

源码 wheel_legged_vmc_flat_env.py 第 567 至 592 行

def _get_heading_frame_horizontal_velocity(self) -> torch.Tensor:

"""Return horizontal velocity in the yaw-aligned heading frame.

Unlike ``root_lin_vel_b[:, :2]``, this projection is independent of

body roll and pitch. The policy therefore cannot change its measured

forward speed merely by leaning the chassis.

"""

forward_axis_b = torch.zeros(self.num_envs, 3, device=self.device)

forward_axis_b[:, 0] = 1.0

forward_axis_w = quat_apply(self._robot.data.root_quat_w,

forward_axis_b)

heading_forward = forward_axis_w[:, :2]

heading_forward = heading_forward / torch.clamp(

torch.linalg.vector_norm(heading_forward, dim=1, keepdim=True),

min=1.0e-6,

)

heading_left = torch.stack(

(-heading_forward[:, 1], heading_forward[:, 0]), dim=1

)

velocity_xy_w = self._robot.data.root_lin_vel_w[:, :2]

return torch.stack(

(

torch.sum(velocity_xy_w * heading_forward, dim=1),

torch.sum(velocity_xy_w * heading_left, dim=1),

),

dim=1,

)

机身水平速度从世界速度投影到 yaw 航向基底，避免身体倾斜改变所测前进速度。轮速与机身线速度同时存在，不能在打滑或加速时简单互相代替。轮累计角无界，不作为稳定的策略输入。

# 2 命令更新和抽样的先后

源码 wheel_legged_vmc_flat_env.py 第 603 至 619 行

def _update_commands(self):

"""Resample commands for environments that reached the resampling

time."""

if self.cfg.commands.grouped_training and

self.cfg.commands.ranges_lin_vel_x[0] != self.cfg.commands.ranges_lin_vel_x[1]:

phase = self.common_step_counter // max(1,

int(self.cfg.commands.resampling_time / self.step_dt))

if phase != self._last_grouped_command_phase:

self._last_grouped_command_phase = phase

# 固定站立组保持零速，其余任务组按全局阶段轮换，并一起重采样。

self._resample_commands_for(torch.arange(self.num_envs,

device=self.device))

return

env_ids = (

self.episode_length_buf

% int(self.cfg.commands.resampling_time / self.step_dt)

== 0

).nonzero(as_tuple=False).flatten()

if len(env_ids) == 0:

return

self._resample_commands_for(env_ids)

源码 wheel_legged_vmc_flat_env.py 第 621 至 623 行

def _resample_commands_for(self, env_ids):

# 长抽样流程在功能文件中；仍只修改当前环境的同一份命令缓存。

resample_commands(self, env_ids)

源码 env_commands.py 第 12 至 72 行

def resample_commands(self, env_ids):

env_ids = torch.as_tensor(env_ids, dtype=torch.long, device=self.device)

if env_ids.numel() == 0:

return

previous_speed = self._commands[env_ids, 0].clone()

lin_min = float(self._command_ranges[0, 0].item())

lin_max = float(self._command_ranges[0, 1].item())

# 固定速度范围用于回放；按原来的随机数顺序抽取 yaw 和高度。

if lin_min == lin_max:

rand = torch.rand(env_ids.numel(), 3, device=self.device)

self._commands[env_ids, :3] = (

self._command_ranges[:, 1] - self._command_ranges[:, 0]

)[None, :] * rand + self._command_ranges[:, 0][None, :]

else:

step = float(self.common_step_counter)

standing_steps = float(self.cfg.commands.standing_only_steps)

ramp_steps = max(float(self.cfg.commands.motion_ramp_steps), 1.0)

progress = min(max((step - standing_steps) / ramp_steps, 0.0), 1.0)

reverse_progress = min(

max(

(step - self.cfg.commands.reverse_ramp_start_steps)

/ self.cfg.commands.reverse_ramp_steps,

0.0,

),

1.0,

)

# 课程先从零速和中心高度开始，随后扩展到配置的高度范围。

self._commands[env_ids, :3] = 0.0

height_min, height_max = self.cfg.commands.ranges_height

height_center = self.cfg.rewards.base_height_target

progressive_height_min = height_center + (

height_min - height_center

) * progress

progressive_height_max = height_center + (

height_max - height_center

) * progress

self._commands[env_ids, 2] = progressive_height_min + (

progressive_height_max - progressive_height_min

) * torch.rand(env_ids.numel(), device=self.device)

height_sample = torch.rand(env_ids.numel(), device=self.device)

half_boundary_fraction = self.cfg.commands.height_boundary_fraction /

2.0

low_height_ids = env_ids[height_sample < half_boundary_fraction]

high_height_ids = env_ids[

(height_sample >= half_boundary_fraction)

& (height_sample < 2.0 * half_boundary_fraction)

]

self._commands[low_height_ids, 2] = progressive_height_min

self._commands[high_height_ids, 2] = progressive_height_max

# 当前训练只保留任务分组：固定站立组，其余环境轮换五种运动任务。

yaw_progress = (step - self.cfg.commands.yaw_start_steps) /

self.cfg.commands.yaw_ramp_steps

speed, yaw = sample_grouped_motion(

env_ids, self.cfg.commands, progress, reverse_progress,

yaw_progress,

phase=self.common_step_counter // max(1,

int(self.cfg.commands.resampling_time / self.step_dt)),

dedicated_standing_count=self._dedicated_standing_count,

)

self._commands[env_ids, 0] = speed

self._commands[env_ids, 1] = yaw

self._target_lin_vel_x[env_ids] = self._commands[env_ids, 0]

self._commands[env_ids, 0] = previous_speed

抽样得到新的目标速度后，_target_lin_vel_x 保存终点，_commands[:,0] 恢复上一值，再由 _get_observations 每控制步最多推进 acceleration_limit × step_dt。这使 PPO 看到的是应用中的斜坡命令。高度在训练课程中独立采样；键盘高度另有平滑斜坡，不能把手动测试序列强塞成训练固定轨迹。

固定速度范围 min=max 是确定性回放入口，会绕过课程抽样，但原有速度斜坡依然保留。训练课程由 common_step_counter 驱动，续训由 checkpoint iteration × num_steps_per_env 恢复。固定站立 ID 不跟着角色轮换；剩余运动 ID 按共同周期轮换，异步 reset 不改变这些 ID 的任务归属。

heading_command=True 时 yaw 角误差生成 rate 指令，heading_command=False 时直接训练 rate。当前转向训练与验收默认关闭额外航向反馈，避免外部闭环替策略纠偏。仅要求 yaw rate=0 并不等价于保持某个世界航向。

# 3 奖励从信号到最终标量

源码 wheel_legged_vmc_flat_env.py 第 627 至 849 行

def _get_rewards(self) -> torch.Tensor:

self._update_forward_kinematics()

root_lin_vel_b = self._robot.data.root_lin_vel_b

root_ang_vel_b = self._robot.data.root_ang_vel_b

proj_gravity = self._robot.data.projected_gravity_b

joint_pos = self._robot.data.joint_pos

joint_vel = self._robot.data.joint_vel

joint_acc = self._robot.data.joint_acc

torque = self._robot.data.applied_torque

contact_forces = self._contact_sensor.data.net_forces_w

heading_velocity = self._get_heading_frame_horizontal_velocity()

wheel_velocity = self._get_wheel_vel_forward_positive()

# 使用航向对齐的水平速度，避免机身倾斜改变“实际前进速度”的定义。

lin_vel_error = torch.square(

self._commands[:, 0] - heading_velocity[:, 0]

)

lin_vel_penalty = horizontal_velocity_error_squared(

heading_velocity, self._commands[:, 0],

self.cfg.rewards.lin_vel_error_lateral_multiplier,

)

r_track_lin = torch.exp(-lin_vel_error /

self.cfg.rewards.tracking_sigma)

r_track_lin_enhance = (

torch.exp(

-lin_vel_error / self.cfg.rewards.tracking_sigma_enhance

)

- 1.0

)

r_track_lin_precise = torch.exp(

-lin_vel_error / self.cfg.rewards.tracking_sigma_precise

)

standing = (torch.abs(self._commands[:, 0]) < 0.01) & (

torch.abs(self._target_lin_vel_x) < 0.01

)

r_standing_velocity = torch.sum(torch.square(heading_velocity), dim=1) *

standing

r_standing_leg_angle = torch.sum(self._theta0.square(), dim=1) *

standing

wheel_reference = self._actions[:, (2, 5)] * self.cfg.action_scale_vel

r_standing_wheel_tracking = torch.sum(

(wheel_velocity - wheel_reference).square(), dim=1

) * (standing & (self._commands[:, 1].abs() < 0.01))

ang_vel_error = torch.square(self._commands[:, 1] - root_ang_vel_b[:,

2])

r_track_ang = torch.exp(-ang_vel_error /

self.cfg.rewards.tracking_sigma_ang)

r_spin_center = spin_center_velocity_penalty(

wheel_velocity, self.cfg.wheel_radius,

self._commands[:, 0], self._target_lin_vel_x, self._commands[:, 1],

)

base_height_error = torch.square(

self._robot.data.root_pos_w[:, 2] - self._commands[:, 2]

)

if self.cfg.rewards.base_height < 0:

r_base_height = torch.sqrt(base_height_error)

else:

r_base_height = torch.exp(

-base_height_error / self.cfg.rewards.base_height_sigma

)

theta_difference = torch.square(self._theta0[:, 0] - self._theta0[:, 1])

if self.cfg.rewards.nominal_state < 0:

r_nominal_state = theta_difference

else:

r_nominal_state = torch.exp(-theta_difference / 0.1)

r_lin_vel_z = torch.square(root_lin_vel_b[:, 2])

r_ang_vel_xy = torch.sum(torch.square(root_ang_vel_b[:, :2]), dim=1)

# 小角度时投影重力 x 主要反映 pitch，y 主要反映 roll。

r_orientation = torch.square(proj_gravity[:, 0]) + (

self.cfg.rewards.orientation_roll_multiplier

* torch.square(proj_gravity[:, 1])

)

r_dof_vel = torch.sum(

torch.square(joint_vel[:, self._leg_joint_ids]), dim=1

)

r_dof_acc = torch.sum(torch.square(joint_acc), dim=1)

r_torques = torch.sum(torch.square(torque), dim=1)

r_action_rate = torch.sum(

torch.square(self._last_actions[:, :, 0] - self._actions), dim=1

)

r_action_smooth = action_second_difference_penalty(

self._actions, self._last_actions[:, :, 0], self._last_actions[:, :,

1],

self.cfg.rewards.action_smooth_wheel_multiplier,

)

# 软饱和读取执行裁剪前的动作，使超出 ±1 的输出仍能被区分和惩罚。

zero = torch.zeros_like(self._raw_actions[:, 0])

if self.cfg.leg_control_mode == DIRECT_JOINT_POSITION_MODE:

# 直接模式的 1/4 列是关节角动作，不能再套用虚拟腿长度动作惩罚。

r_leg_length_action_saturation = r_leg_length_action_difference =

r_leg_length_target_underreach = zero

raw_joint_actions = self._raw_actions[:,

LEG_ACTION_COLUMNS].clamp(-10., 10.)

r_joint_position_action_saturation = torch.relu(

raw_joint_actions.abs() -

self.cfg.rewards.joint_position_action_soft_limit,

).square().sum(dim=1)

else:

raw_length_actions = self._raw_actions[:, (1, 4)].clamp(-10.0, 10.0)

r_leg_length_action_saturation = torch.relu(

raw_length_actions.abs() -

self.cfg.rewards.leg_length_action_soft_limit,

).square().sum(dim=1)

r_leg_length_action_difference = (self._actions[:, 1] -

self._actions[:, 4]).square()

leg_length_refs = (

self._raw_actions[:, (1, 4)].clamp(-1.0, 1.0) *

self.cfg.action_scale_l0 + self.cfg.l0_offset

)

minimum_leg_length_ref = (

self._commands[:, 2:3] +

self.cfg.rewards.leg_length_target_height_offset

).clamp(max=self.cfg.rewards.leg_length_target_cap)

r_leg_length_target_underreach = torch.relu(

minimum_leg_length_ref - leg_length_refs,

).square().mean(dim=1)

r_joint_position_action_saturation = zero

contact_force_norm = torch.norm(

contact_forces[:, self._penalised_contact_ids, :], dim=-1

)

r_collision = torch.sum(

1.0 * (contact_force_norm > self.cfg.collision_force_threshold),

dim=1

)

dof_pos_limits = self._robot.data.joint_pos_limits

leg_pos = joint_pos[:, self._leg_joint_ids]

leg_pos_limits = dof_pos_limits[:, self._leg_joint_ids]

out_of_limits = -(leg_pos - leg_pos_limits[:, :, 0]).clip(max=0.0)

out_of_limits += (leg_pos - leg_pos_limits[:, :, 1]).clip(min=0.0)

r_dof_pos_limits = torch.sum(out_of_limits, dim=1)

# Keep each signal and its config scale in one auditable table.

signals = {

"track_lin_vel": r_track_lin,

"track_lin_vel_enhance": r_track_lin_enhance,

"track_lin_vel_precise": r_track_lin_precise,

"lin_vel_error_sq": lin_vel_penalty,

"standing_velocity": r_standing_velocity,

"base_height_error_sq": base_height_error,

"track_ang_vel": r_track_ang,

"yaw_rate_error_sq": ang_vel_error,

"spin_center_velocity": r_spin_center,

"base_height": r_base_height,

"nominal_state": r_nominal_state,

"lin_vel_z": r_lin_vel_z,

"ang_vel_xy": r_ang_vel_xy,

"orientation": r_orientation,

"dof_vel": r_dof_vel,

"dof_acc": r_dof_acc,

"torques": r_torques,

"action_rate": r_action_rate,

"action_smooth": r_action_smooth,

"leg_length_action_saturation": r_leg_length_action_saturation,

"leg_length_action_difference": r_leg_length_action_difference,

"leg_length_target_underreach": r_leg_length_target_underreach,

"joint_position_action_saturation":

r_joint_position_action_saturation,

"collision": r_collision,

"dof_pos_limits": r_dof_pos_limits,

}

rewards = {

key: value * getattr(self.cfg.rewards, REWARD_SCALE_FIELDS.get(key,

key)) * self.step_dt

for key, value in signals.items()

}

# 各项负奖励单独设上限，避免姿态等平方代价过早进入通用裁剪平坦区。

reward_bound = self.cfg.rewards.clip_single_reward * self.step_dt

penalty_bounds = {

"nominal_state": self.cfg.rewards.nominal_state_penalty_clip *

self.step_dt,

"orientation": self.cfg.rewards.orientation_penalty_clip *

self.step_dt,

"base_height_error_sq": self.cfg.rewards.base_height_penalty_clip *

self.step_dt,

"standing_leg_angle":

self.cfg.rewards.standing_leg_angle_penalty_clip * self.step_dt,

"lin_vel_error_sq": self.cfg.rewards.lin_vel_penalty_clip *

self.step_dt,

"yaw_rate_error_sq": self.cfg.rewards.yaw_rate_penalty_clip *

self.step_dt,

"standing_wheel_tracking":

self.cfg.rewards.standing_wheel_penalty_clip * self.step_dt,

"action_rate": self.cfg.rewards.action_rate_penalty_clip *

self.step_dt,

"action_smooth": self.cfg.rewards.action_smooth_penalty_clip *

self.step_dt,

}

rewards["standing_leg_angle"] = r_standing_leg_angle *

self.cfg.rewards.standing_leg_angle * self.step_dt

rewards["standing_wheel_tracking"] = r_standing_wheel_tracking *

self.cfg.rewards.standing_wheel_tracking * self.step_dt

if (

not self._tracking_diagnostics or self.common_step_counter % 100 ==

0

):

# 记录实际应用命令与速度；诊断统计不参与动作执行或奖励求和。

diagnostics={}

opposite = (self._commands[:, 0] * heading_velocity[:, 0] < 0) &

(heading_velocity[:, 0].abs() > .05)

for name,mask in (

('standing',standing & (self._commands[:,1].abs()<.01)),

('forward',self._commands[:,0]>.05),

('reverse',self._commands[:,0]<-.05),

('spin',standing & (self._commands[:,1].abs()>.05)),

):

count=mask.sum().clamp_min(1)

diagnostics[f'Tracking/{name}_fraction']=mask.float().mean().item()

for label,value in (

('command_m_s',self._commands[:,0]),

('speed_m_s',heading_velocity[:,0]),

('speed_mae_m_s',(self._commands[:,0]-heading_velocity[:,0]).abs()),

('yaw_mae_rad_s',(self._commands[:,1]-root_ang_vel_b[:,2]).abs()),

):

diagnostics[f'Tracking/{name}_{label}']=((value*mask).sum()/count).item()

diagnostics[f'Tracking/{name}_opposite_direction_fraction'] =

((opposite * mask).sum() / count).item()

diagnostics['Tracking/wheel_reference_delta_rms_rad_s']=torch.sqrt(

(self._actions[:,(2,5)]-self._last_actions[:,(2,5),0]).square().mean()

).item()*self.cfg.action_scale_vel

diagnostics['Tracking/wheel_reference_second_difference_rms_rad_s']

= torch.sqrt(

(self._actions[:, (2, 5)] - 2 * self._last_actions[:, (2, 5), 0]

+ self._last_actions[:, (2, 5), 1]).square().mean()

).item() * self.cfg.action_scale_vel

for key in

('lin_vel_error_sq','yaw_rate_error_sq','standing_wheel_tracking','action_rate','action_smooth','orientation'):

bound=penalty_bounds[key]

diagnostics[f'Reward_Clipping/{key}']=(rewards[key]<=-bound).float().mean().item()

self._tracking_diagnostics = diagnostics

rewards = {

key: torch.nan_to_num(

torch.clamp(

value,

min=-penalty_bounds.get(key, reward_bound),

max=reward_bound,

),

nan=0.0,

posinf=0.0,

neginf=0.0,

)

for key, value in rewards.items()

}

# LAB 先计算 reset_terminated；不安全终止只扣一次，超时不扣此项。

rewards["termination"] = self.reset_terminated.float() *

self.cfg.rewards.termination

self._last_reward_terms = {

key: value.detach() for key, value in rewards.items()

}

reward = torch.sum(torch.stack(list(rewards.values())), dim=0)

for key, value in rewards.items():

self._episode_sums[key] += value

return reward

每个信号先由真实状态与应用命令计算，再乘 cfg.rewards 对应权重和 dt，按逐项上限裁剪，最后求和并累积日志。终止代价最后单独加入，只对不安全结束生效。Episode_Reward 日志除以固定回合时长，不是这个时刻策略直接收到的 reward，也不直接表示实际速度误差。

| 信号 | 含义 | 理解要点 |
| --- | --- | --- |
| lin_​vel_​error_​sq | (vx_​cmd−vx)² + multiplier×vy² | 覆盖侧漂；单位为速度平方 |
| yaw_​rate_​error_​sq | (yaw_​rate_​cmd−gyro_​z)² | 约束角速度，不直接约束 yaw 角 |
| base_​height | exp(−(z−h_​cmd)² /​ sigma) | z 为根节点高度，m；sigma 单位 m² |
| orientation | gravity_​x² + roll_​multiplier×gravity_​y² | 姿态目标水平，不要求腿长相同 |
| joint_​position_​action_​saturation | Σrelu(¦raw_​leg_​action¦−0.85)² | 读 raw，允许临时大动作但不鼓励长期越界 |
| action_​rate | Σ(action−last_​action)² | 对执行参考的一阶变化，不等于罚非零平衡力矩 |
| collision /​ termination | 非轮接触计数／一次事故代价 | 持续计时和事故类型另在 _​get_​dones |

默认关闭的项仍定义在唯一 RewardsCfg 中，以便有记录的对照；不再有看似 -150 最终被另一个 profile 改为 -20 的隐式覆盖。修改权重和核宽度的结果需要训练才能体现，单改 reward 后冻结旧策略不会立即改变动作。

# 4 终止条件和持续计数

源码 wheel_legged_vmc_flat_env.py 第 910 至 917 行

def _advance_failure_counter(self, counter, unsafe, seconds, *,

inclusive=False, minimum_steps=None):

"""Count continuous unsafe steps; a safe step clears only this timer."""

counter = torch.where(unsafe, counter + 1.0, 0.0)

threshold = seconds / self.step_dt

if minimum_steps is not None:

threshold = max(minimum_steps, threshold)

died = counter >= threshold if inclusive else counter > threshold

return counter, died

源码 wheel_legged_vmc_flat_env.py 第 853 至 908 行

def _get_dones(self) -> tuple[torch.Tensor, torch.Tensor]:

contact_forces = self._contact_sensor.data.net_forces_w

gravity = self._robot.data.projected_gravity_b

base_contact = torch.norm(contact_forces[:, self._base_id, :],

dim=-1).gt(

self.cfg.base_contact_force_threshold,

).any(dim=1)

self._base_contact_buf, base_died = self._advance_failure_counter(

self._base_contact_buf, base_contact,

self.cfg.base_contact_terminal_time_s,

)

self._fail_buf, fallen_died = self._advance_failure_counter(

self._fail_buf, gravity[:, 2] > -0.1,

self.cfg.fail_to_terminal_time_s,

)

body_pitch = torch.asin(gravity[:, 0].clamp(-1.0, 1.0)).abs()

self._pitch_fail_buf, pitch_died = self._advance_failure_counter(

self._pitch_fail_buf, body_pitch > self.cfg.max_body_pitch,

self.cfg.body_pitch_terminal_time_s,

)

body_roll = torch.asin(gravity[:, 1].clamp(-1.0, 1.0)).abs()

self._roll_fail_buf, roll_died = self._advance_failure_counter(

self._roll_fail_buf, body_roll > self.cfg.max_body_roll,

self.cfg.body_roll_terminal_time_s, inclusive=True,

)

# Only wheels may maintain ground contact. Brief impacts are tolerated.

leg_contact = torch.norm(contact_forces[:, self._leg_contact_ids, :],

dim=-1).gt(

self.cfg.leg_contact_force_threshold,

).any(dim=1)

self._leg_contact_buf, leg_died = self._advance_failure_counter(

self._leg_contact_buf, leg_contact,

self.cfg.leg_contact_terminal_time_s,

)

# Height also catches a collapsed chassis with intermittent force

readings.

self._low_height_buf, low_height_died = self._advance_failure_counter(

self._low_height_buf, self._robot.data.root_pos_w[:, 2] <

self.cfg.min_root_height,

self.cfg.low_height_terminal_time_s, inclusive=True,

minimum_steps=1.0,

)

numerical_failure = ~(

torch.isfinite(self._robot.data.root_pos_w).all(dim=1)

& torch.isfinite(self._robot.data.root_quat_w).all(dim=1)

& torch.isfinite(self._robot.data.root_lin_vel_w).all(dim=1)

& torch.isfinite(self._robot.data.root_ang_vel_w).all(dim=1)

& torch.isfinite(self._robot.data.joint_pos).all(dim=1)

& torch.isfinite(self._robot.data.joint_vel).all(dim=1)

& torch.isfinite(contact_forces).all(dim=(1, 2))

)

self._termination_reasons = {

"base_contact": base_died,

"fallen": fallen_died,

"pitch": pitch_died,

"roll": roll_died,

"leg_contact": leg_died,

"numerical": numerical_failure,

"kinematic_singularity": ~self._five_bar_valid.all(dim=1),

"low_height": low_height_died,

}

died = torch.stack(tuple(self._termination_reasons.values())).any(dim=0)

time_out = self.episode_length_buf >= self.max_episode_length - 1

return died, time_out

每种事故有自己的计数器，只在对应状态持续不安全时增长，恢复安全即归零。严格 > 与 >= 在 20 ms 网格上不同：机身 0.15 s 接触通常第 8 步终止，连杆 0.2 s 接触通常第 11 步；roll 0.2 s 使用 >=，第 10 步。pitch、低高度和数值/奇异故障分别记录，不能把 timeout 当作跌倒。

净接触力可能包括机械限位自接触。读 body ID 不能混用 joint ID；单位 N 与角度 rad、速度 rad/s 不可互换。诊断基座到底碰了地还是止挡时，应看过滤后的地面接触与几何间隙，而不是直接取消安全规则。

# 5 如何检查数据而不扰动观测

_get_observations 同时推进命令、航向反馈和动作历史，并不是无副作用 getter。同一个控制步不要为了调试再调用一次，否则可能改变 last_actions 和命令斜坡。使用脚本记录真正送入策略的输入与 raw mean，故障状态在自动 reset 前记录，才不会把重置后的漂亮姿态误当作控制正常。

重点对照静止、正反速度、双向 yaw 与高低高度；看速度 MAE、yaw rate MAE、实际高度误差、pitch/roll、左右实际 L0/摆角、应用目标角、raw 动作和饱和比例。零目标采用有单位的绝对容差，不能以 0 做百分比的分母。当前矩阵无重置证明存活，不代表所有精度已经达标。

# 源码索引和版本核对

wheel_legged_isaaclab/wheel_legged_gym_isaaclab/tasks/direct/wheel_legged_vmc_flat/env_commands.py

wheel_legged_isaaclab/wheel_legged_gym_isaaclab/tasks/direct/wheel_legged_vmc_flat/wheel_legged_vmc_flat_env.py

版本核对不应只看文件名。模型应携带 params/env.yaml 和 params/agent.yaml；训练分支的覆盖值以配套配置为准。本教程解释源码快照，不作为随训练动态变化的实时监控面板。